13/10/2026                                                                                                                Lathika S

192425066

ITA0329 – Mobile Computing

CO1-ASSESSMENT TOOL - 1

Q1.

A full-duplex channel uses two simplex channels.
Number of channels = 45,000 kHz ÷ (2 × 25 kHz) = 900.
                     =900 full-duplex channels

Q2.

Channels per cell = 560 ÷ 7 = 80.

=80 channels per cell.

Q3.

Total voice channels = 100 × 8 = 800.
                    =800 simultaneous voice channels.

Q4.

Power ratio in dB = 10 log₁₀(80 ÷ 4) = 10 log₁₀(20) ≈ 13.01 dB.
                    =13.01 dB.

Q5.

C/I = −60 − (−92) = 32 dB.
                    =32 dB.

Q6.

Total channels = 42,000 kHz ÷ 300 kHz = 140.
Channels per cell = 140 ÷ 7 = 20.
                 =20 channels per cell.

Q7.

With path-loss exponent n = 4, doubling distance reduces power by 2⁴ = 16.
Received power = 64 W ÷ 16 = 4 W.
4 W, assuming 64 W is the received power at the original distance.

Q8.

For inverse-square propagation, power is proportional to distance².
Power factor = 2² = 4.
Increase transmit power by a factor of 4.

Q9.

Total channels = 4 × 15 = 60.
                  =60 channels.

Q10.

Speed = 100 ÷ 3.6 = 27.78 m/s.
Maximum Doppler shift = vf/c = (27.78 × 900 × 10⁶) ÷ (3 × 10⁸) ≈ 83.3 Hz.
                   =83.3 Hz (maximum magnitude).

Q11.

Slot duration = 5 ms ÷ 8 = 0.625 ms.
                   =0.625 ms.

Q12.

Number of carriers = 24,000 kHz ÷ 200 kHz = 120.
                   =120 carriers.

Q13.

Transmit power = 10 log₁₀(8,000 mW) ≈ 39.03 dBm.
Received power = 39.03 − 33 = 6.03 dBm.
                     = 6.03 dBm.

Q14.

FSPL (dB) = 32.44 + 20 log₁₀(f in MHz) + 20 log₁₀(d in km).
= 32.44 + 20 log₁₀(1900) + 20 log₁₀(2.5) ≈ 105.98 dB.
                      =106 dB.

Q15.

Cluster capacity = 6 × 250 = 1,500.
                     =1,500 simultaneous calls.

Q16.

Using Little’s Law: Active calls = Arrival rate × Average duration.
= 4,000 calls/s × 180 s = 720,000 calls.
               =720,000 active calls, assuming the arrival rate is sustained.

Q17.

Bandwidth = 150 × 25 kHz = 3,750 kHz = 3.75 MHz.
                =3.75 MHz.

Q18.

There are 29 gaps between 30 adjacent bands.
Guard-band bandwidth = 29 × 40 kHz = 1,160 kHz.
                        =1.16 MHz. (If 30 guard bands are intended, it is 1.2 MHz.)

Q19.

Cell diameter = 2 × 1.5 = 3 km.
Handoffs per hour ≈ 90 km ÷ 3 km = 30.

Approximately 30 handoffs per hour.

Q20.

Total capacity = 8 × 40 × 250 = 80,000.
                       =80,000 users.

Q21.

Radius = 400 m = 0.4 km.
Approximate cell area = R² = 0.4² = 0.16 km².
Number of cells = 400 ÷ 0.16 = 2,500.
                    =Approximately 2,500 cells.

Q22

Total bandwidth = 6,000 kHz.
Number of channels = 6,000 ÷ 30 = 200.
                      =200 channels.

Q23.

Traffic (Erlangs) = Subscribers × Calls per hour × Call duration (minutes) ÷ 60.
= 1,200 × 4 × 2.5 ÷ 60 = 200 Erlangs.
                     =200 Erlangs.

Q24.

Number of users = 40,000 kHz ÷ 400 kHz = 100.
                      =100 simultaneous users.

Q25.

Total capacity = 1,200 × 600 = 720,000.
                     =720,000 simultaneous users.